AmirAbbas MoumeniZadeh – SID:810101529 – CA1 Report – Digital System ECE 894

- 2-Input Nor Implimentation

According to Calculation, worst-case dalay of this Circuit is 10ns, Which can be shown in the waveform diagram.

Calculation : pMOS Delay: (5,6,7) , nMOS Delay: (3,4,5)

TO1 :2×pMOS to1 , nMOS to z=10 , 5 →WC Delay :10

To0 : 2×pMOS toZ , nMOS to 0=14 , 4 →WC Delay :14

Testbench Code :

`timescale 1ns/1ns

module TBNor();

reg aa,bb;

wire ww;

mynor UUT(.a(aa),.b(bb),.w(ww));

initial begin

aa=0; bb=0;

#100 aa=1 ; bb=0;

#100 aa=1 ; bb=1;

#100 aa=0 ; bb=1;

#100 aa=0 ; bb=0;

#100 $stop;

end

endmodule

Implimentation Code :

`timescale 1ns/1ns

module mynor(input a,b , output w);

wire j;

supply0 gnd;

supply1 vdd;

pmos #(5,6,7)(j,vdd,a);

pmos #(5,6,7)(w,j,b);

nmos #(3,4,5)(w,gnd,a);

nmos #(3,4,5)(w,gnd,b);

endmodule

- 2-to-1 Mux implimentation:

`timescale 1ns/1ns

module twoMuxTB();

reg aa,bb,ss;

wire ww;

twoOneMUX UUT1(.a(aa),.b(bb),.s(ss),.w(ww));

initial begin

#100 ss=0; aa=0; bb=0;

#100 ss=0; aa=0; bb=1;

#100 ss=0; aa=1; bb=1;

#100 ss=0; aa=1; bb=0;

#100 ss=1; aa=0; bb=0;

#100 ss=1; aa=0; bb=1;

#100 ss=1; aa=1; bb=1;

#100 ss=1; aa=1; bb=0;

end

endmodule

We can see through the waveform that in first half of wave form which S is 0, output just looks at a and follows it and in the second half the opposite. Which means when S is 1, output follows b.

`timescale 1ns/1ns

module twoOneMUX(input a,b,s , output w);

wire j;

supply0 gnd;

supply1 vdd;

nmos #(3,4,5)(w,a,s);

pmos #(5,6,7)(j,vdd,s);

nmos #(3,4,5)(j,gnd,s);

nmos #(3,4,5)(w,b,j);

endmodule

`timescale 1ns/1ns

module twoOneMUX(input a,b,s , output w);

wire j;

supply0 gnd;

supply1 vdd;

nmos #(3,4,5)(w,a,s);

pmos #(5,6,7)(j,vdd,s);

nmos #(3,4,5)(j,gnd,s);

nmos #(3,4,5)(w,b,j);

endmodule

worst case delay :

to1 :nMOS toZ+pMOS toZ=5+7=12

to0 :nMOS toZ+pMOS toZ=5+7=12

- 4-to-1 Mux

`timescale 1ns/1ns

module fourMuxTB();

reg aa,bb,cc,dd,ss1,ss0;

wire ww;

fourOneMUX UUT1(.a(aa),.b(bb),.c(cc),.d(dd),.s1(ss1),.s0(ss0),.w(ww));

initial begin

#100 ss1=0; ss0=0; aa=1; bb=0; cc=0; dd=0;

#100 ss1=0; ss0=1; aa=0; bb=1; cc=0; dd=0;

#100 ss1=1; ss0=1; aa=0; bb=0; cc=0; dd=1;

#100 ss1=1; ss0=0; aa=0; bb=0; cc=1; dd=0;

#100 ss1=0; ss0=0; aa=0; bb=1; cc=1; dd=1;

#100 ss1=0; ss0=1; aa=1; bb=0; cc=1; dd=1;

#100 ss1=1; ss0=1; aa=1; bb=1; cc=1; dd=0;

#100 ss1=1; ss0=0; aa=1; bb=1; cc=0; dd=1;

end

endmodule

`timescale 1ns/1ns

module fourOneMUX(input a,b,c,d,s1,s0 , output w);

wire i,j,k,l,p;

nor #(15,21)(i,~s1,~s0,a);

nor #(15,21)(j,~s1,s0,b);

nor #(15,21)(k,s1,~s0,c);

nor #(15,21)(l,s1,s0,d);

nor #(20,28)(p,i,j,k,l);

nor #(10,14)(w,p,p);

endmodule

we can see through waveform that s1 and s0 are controling the output. When we have 00 on control we see a on output. 01 for b, 10 for c and 11 for d. ( the testbench is made in a situation that in every step the value that we expect to have it on output is different from the others so easily can be undrestood that circuit Is working correct or not).

Worst Case Delay : 15+20+2×10=

55

Which makes sence bacause we have a 3-input and a four input Nor and two invertors which have been made by conecting Nors inputs together SO we expect to have a lot of Timing delay

- 4-to-1 Mux Implimenting with 2-to-1 Mux

Test bench is just like the previous question. we can see through waveform that s1 and s0 are controling the output. When we have 00 on control we see a on output. 01 for b, 10 for c and 11 for d.

`timescale 1ns/1ns

module QuesFourMUX(input a,b,c,d,s1,s0 , output w);

wire i,j,s0bar , s1bar;

supply0 gnd;

supply1 vdd;

//Mux1 a,b

nmos #(3,4,5)(i,b,s0);

nmos #(3,4,5)(i,a,s0bar);

//s0 inverter

nmos #(3,4,5)(s0bar,gnd,s0);

pmos #(5,6,7)(s0bar,vdd,s0);

//Mux2 c,d

nmos #(3,4,5)(j,c,s0bar);

nmos #(3,4,5)(j,d,s0);

//Mux3

nmos #(3,4,5)(w,j,s1);

nmos #(3,4,5)(w,i,s1bar);

//s1 inverter

nmos #(3,4,5)(s1bar,gnd,s1);

pmos #(5,6,7)(s1bar,vdd,s1);

endmodule

`timescale 1ns/1ns

module fourMuxTB();

reg aa,bb,cc,dd,ss1,ss0;

wire ww;

fourOneMUX UUT1(.a(aa),.b(bb),.c(cc),.d(dd),.s1(ss1),.s0(ss0),.w(ww));

initial begin

#100 ss1=0; ss0=0; aa=1; bb=0; cc=0; dd=0;

#100 ss1=0; ss0=1; aa=0; bb=1; cc=0; dd=0;

#100 ss1=1; ss0=1; aa=0; bb=0; cc=0; dd=1;

#100 ss1=1; ss0=0; aa=0; bb=0; cc=1; dd=0;

#100 ss1=0; ss0=0; aa=0; bb=1; cc=1; dd=1;

#100 ss1=0; ss0=1; aa=1; bb=0; cc=1; dd=1;

#100 ss1=1; ss0=1; aa=1; bb=1; cc=1; dd=0;

#100 ss1=1; ss0=0; aa=1; bb=1; cc=0; dd=1;

end

endmodule

We can see that there is a lot of glitch on output comparing to the previous question but the timing delay is lower than previous one. We can examine this in next question in next question more accurate with seeing both outputs together.

- Examine both 4-to-1 Muxes together

ww1 is output for Mux made with Nor Gates and ww2 is output for 4-1Mux made with 3 2-1 Muxes. As we can see delay for ww1 is much more than ww2 which is bacause in first circuit there are much more transistors in use (≅32 switches) but in second circuit this number is much lower (10 Switches)

Hand drawn waveform for Problem 1.